/*

* ============================================================

*  rice_vendo_esp32.ino

* ============================================================

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ESP32Servo.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ---- WiFi ----

const char* ssid     = "ANDREI PISOWIFI 6G";

const char* password = "12345678";

// ---- Server ----

const char* serverUrl = "https://ricevendo.minsualumni.site/api.php";

// ---- Settings ----

const int REQUIRED = 58;

const int DISPENSE_MS = 3000;

// ---- Pins ----

#define COIN_PIN   4

#define SERVO_PIN  18

#define BUTTON_PIN 5

// ---- LCD ----

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ---- Servo ----

Servo myServo;

// ---- State ----

volatile int pulses = 0;

volatile unsigned long lastPulseTime = 0;

int money = 0;

unsigned long lastPingMs = 0;

// ============================================================

// COIN INTERRUPT

// ============================================================

void IRAM_ATTR onCoinPulse() {

unsigned long now = millis();

if (now - lastPulseTime > 25) {

pulses++;

lastPulseTime = now;

}

}

// ============================================================

// SEND HTTP JSON

// ============================================================

bool sendPost(String jsonBody) {

if (WiFi.status() != WL_CONNECTED) return false;

WiFiClientSecure client;

client.setInsecure(); // IMPORTANT FIX for HTTPS

HTTPClient http;

http.begin(client, serverUrl);

http.addHeader("Content-Type", "application/json");

int code = http.POST(jsonBody);

String response = http.getString();

Serial.printf("HTTP %d: %s\n", code, response.c_str());

http.end();

return (code == 200);

}

// ============================================================

void sendCoinInsert(int amount) {

String body = "{\"action\":\"coin_insert\",\"amount\":" + String(amount) + "}";

sendPost(body);

}

void sendDispense(int amount) {

String body = "{\"action\":\"dispense\",\"amount\":" + String(amount) + "}";

sendPost(body);

}

void sendPing() {

sendPost("{\"action\":\"ping\"}");

}

// ============================================================

// SETUP

// ============================================================

void setup() {

Serial.begin(115200);

// LCD

Wire.begin(21, 22);

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Rice Vendo");

lcd.setCursor(0, 1);

lcd.print("Starting...");

// Pins

pinMode(COIN_PIN, INPUT_PULLUP);

pinMode(BUTTON_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(COIN_PIN), onCoinPulse, FALLING);

// Servo

myServo.attach(SERVO_PIN);

myServo.write(0);

// WiFi

WiFi.begin(ssid, password);

int tries = 0;

while (WiFi.status() != WL_CONNECTED && tries < 30) {

delay(500);

Serial.print(".");

tries++;

}

lcd.clear();

if (WiFi.status() == WL_CONNECTED) {

lcd.setCursor(0, 0);

lcd.print("WiFi Connected");

lcd.setCursor(0, 1);

lcd.print(WiFi.localIP());

} else {

lcd.setCursor(0, 0);

lcd.print("No WiFi");

lcd.setCursor(0, 1);

lcd.print("Offline Mode");

}

delay(2000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Insert P58");

}

// ============================================================

// LOOP

// ============================================================

void loop() {

// ===== PROCESS COINS =====

static unsigned long lastCoinProcess = 0;

if (millis() - lastCoinProcess > 250 && pulses > 0) {

noInterrupts();

int counted = pulses;

pulses = 0;

interrupts();

money += counted;

lastCoinProcess = millis();

sendCoinInsert(counted);

lcd.setCursor(0, 0);

lcd.print("Amount: P");

lcd.print(money);

lcd.print("   ");

lcd.setCursor(0, 1);

if (money >= REQUIRED) {

lcd.print("Press Button   ");

} else {

lcd.print("Need P");

lcd.print(REQUIRED - money);

lcd.print("   ");

}

}

// ===== DISPENSE =====

if (digitalRead(BUTTON_PIN) == LOW && money >= REQUIRED) {

delay(50);

if (digitalRead(BUTTON_PIN) == LOW) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Dispensing...");

myServo.write(90);

delay(DISPENSE_MS);

myServo.write(0);

sendDispense(REQUIRED);

money -= REQUIRED;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Done!");

delay(1500);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Amount: P");

lcd.print(money);

lcd.setCursor(0, 1);

if (money >= REQUIRED) {

lcd.print("Press Button");

} else {

lcd.print("Insert P58");

}

while (digitalRead(BUTTON_PIN) == LOW) {

delay(10);

}

}

}

// ===== HEARTBEAT =====

if (millis() - lastPingMs > 30000) {

lastPingMs = millis();

sendPing();

}

delay(10);

}